Digital Audio Matrix Processor

SoniX M8800

User's manual

Ver1.1.9

Contents

1. Version update instructions 	4

2. Important Note	 4

3. Safety information	 4

4. Product introduction	 6

5. Performance parameters	 7

6. Control software running environment	 8

7. Control software connect to device	 8

8. Password Login 	9

9. Reset Password login settings 	10

10. IP Setting  	11

11. Device List Bar  	12

12. Device Information Status Bar 	 13

13. Input Setup	14

14. Feedback suppressor	14

15. AGC--- Automatic gain 	15

16. Gate--- Noise gate 	16

17. Input Filter	 16

18. Input Limiter  	17

19. Pre Mix volume 	18

20. Matrix mixing 	18

21. Auto mixing 	18

22. Signal Generator 	19

23. Output Level   	20

24. Output Filter	 20

25. Output Limiter	 21

26. Output Delay	 21

27. Preset 	22

28. Setup 	23

29. Recording/ Play 	 24

30. Label 	25

31. Meter Level	 26

32. Device List  	 26

33. Command Assistant 	27

34. Third-party communication protocol-ASCII character format 	28

35. Third-party communication protocol: TCP HEX format	 31

35.1 Data sending format 	31

35.2 Device return data format 	31

35.3 Example 	32

36. Mixing matrix TCP/IP, RS-232 control instructions	 34

36.1 Control the mixing status (Function ID: 01H [R/W]) 	34

36.2 Control the mixing value (Function ID: 02H [R/W]) 	35

37. Noise gate coding and camera linkage	 37

38. System Upgrade 	38

39. System Reset 	38

40. Common Troubleshooting 	38

40.1 The control software cannot find the processor 	38

40.2 The control software finds the device but cannot establish a connection 	38

40.3 PC control software cannot modify the IP address	 38

40.4 Third-party TCP commands cannot be controlled	 39

# Version update instructions

| Update Date | ARM version | Update content |
| --- | --- | --- |
| 2022.02.26 | V1.0. 0 | Generation based on PC control software v1.0. 7 |
| 2022.10.10 | V1.0.15 | Bug repair |
| 2023.07.08 | V1.1.1 | Configurable language |
| 2023.11.01 | V1.1.14 | Configurable language bug repair |
| 2023.11.10 | V1.1.15 | Configurable language repair |

# Important notes

Please read this operation manual before use. This operation manual involves the safety of equipment use and warranty clauses, so as to guard against equipment damage caused by improper operation.

# Security information

- Please read the following notes carefully and install and use this set of products as required to avoid your loss

Warning: This equipment is installed indoors or used in the cabinet, and shall not be splashed or sprayed by liquid, and shall not put the objects containing liquid on this set of equipment. Never let liquid splash into any part of the system;
Warning:In case of lightning, please unplug the power cable of the equipment to prevent the equipment from being damaged by lightning

Warning:When the equipment is not used for a long time, attention should be paid to the moisture-proof equipment. It is recommended to electrify for 3 hours every week;

Note:

- whole machine power supply:whole machine power supply must be in the voltage range of 100V ~ 240V;

- power cord protection:properly wiring to avoid being trampled or squeezed by heavy objects.

- Maintenance:All maintenance shall be carried out by certified maintenance personnel. In order to avoid the danger of electric shock, do not try to open the equipment and repair it yourself.

- Ventilation Hole:Some equipment have ventilation slots or holes in the casing, which are used to prevent overheating of sensitive elements in the machine. Don't block the vent with anything.

Power related content:

- Please be sure to plug the plug tightly and connect the cable firmly, otherwise it may cause failure;

- Do not use loose power socket or damaged power cord, otherwise there is a danger of electric shock or fire;

- Never touch the power plug with wet hands under any circumstances, otherwise there is a danger of electric shock;

- Please do not plug the power plugs of multiple devices into one socket, otherwise it will cause fire danger;

- Do not press heavy objects on the power cord to prevent bending, pulling or winding the power cord;

- Additional security information:

- Users must read and understand all safety and use instructions before using the package equipment;

- Users should keep safety instructions for future use;

# Product introduction

Front and rear panels

The new digital audio processor is designed with a new hardware architecture, a high-performance 400 MHz floating-point DSP processor, dual-core 1.2G main control chip, linux core, an ultra-low floor noise preamplifier circuit, low distortion analog circuit, and ultra-low floor noise of-100dBu when no audio signal is input, bringing a quieter user experience to the project application.

The front and rear panels have two independent USB interfaces, which can be used for USB recording and playback operations.

A fault alarm indicator lamp on the front panel can flash a red alarm prompt when there is a serious error.

DSP processor capability

It has 400Mhz DSP processing capability, built-in independent 8-channel adaptive feedback suppressor, 8-channel noise gate, 8-channel input limiter, 8-channel 8-band PEQ, 8-channel input 48dB slope high & low pass filter, 8x8 full matrix mixing, and realizes free mixing of any input channel. Eight output channels have independent 8-band PEQ, limiter, 48dB slope high-pass-low-pass, and eight 300-second Delay in output channels.

Built-in sine wave, pink noise and white noise, signal generator. 32 presets can be stored.

Rich audio channels and control interfaces

1 TCP/IP communication port, 1 RS-232 communication port, open third-party control protocol. It can meet the use requirements of various medium and small professional audio projects.

1 RS485 interface dedicated to wall touch screen

Front panel status display

The front panel has 1 power indicator, 1 Ethernet networking indicator and 1 error prompt lamp.

Easy-to-use control software

Simple and easy-to-use software interface, easy to understand, and can be operated quickly without instructions. Each input value can be entered directly on the keyboard to get the exact value you want, such as -12. 2dB directly input-12. 2. Click the volume putter to select the channel, and press the keyboard up/right, down/left direction keys to achieve 0.1 dB step and step reduction. For complex parameter adjustment such as PEQ and Limiter, parameters can be copied and pasted quickly, and you can easily realize multi-channel data replication with convenient operation.

Open RS-232, TCP/IP communication protocol

The third-party equipment can control the volume, call the mode and set the mute, and can read the level meter before mixing and the level meter after mixing in batches through TCP/IP protocol, which is convenient for the third-party software integration.

Wall touch screen volume control panel

Through RS485 interface, externally connected with up to 16 wall touch screen volume control panels, mute and volume adjustment can be realized. The upper limit and lower limit of the volume can be set on the PC software side to prevent the user control from exceeding the use range;

Product features:

- High performance floating-point DSP processing chip

- 8-channel balanced input audio channel

- All input channels support MIC input

- All input channels support 48V phantom power supply

- 8 Balanced Audio Output Channels

- Independent adaptive feedback suppressor of each channel

- Input per channel: 48V, preamplifier, noise gate, limiter, 8-band PEQ, high-pass filter, low-pass filter

- Output per channel: 8-band PEQ, high and low pass filter, limiter, delay (300ms)

- Built-in signal generator: sine wave signal, pink noise, white noise

- 3 LED Status Indicators on Front Panel

- Open RS-232, TCP/IP protocol to realize third-party control

- It has camera tracking code output, which is convenient to realize camera linkage function through third-party central control

- Supports 32 presets, which can be called through TCP/IP and RS-232 protocols

- Built-in USB broadcasting and recording functions

- Independent USB interface for front and rear panels

# Performance parameters

| DSP Frequency, Processing Capability | 400Mhz DSP |
| --- | --- |
| Sampling frequency/quantization | 48 kHz, 24Bit ADC, 24Bit DAC |
| Input to Output Dynamic Range | 110 dB |
| THD | < 0.004% @ 4dBu 20Hz ~ 20kHz |
| Frequency response | 20 Hz-20 kHz, ± 0.5 dB @ 0dBu |
| Common mode rejection ratio (CMRR) | > 80dB @ 1 kHz MIC Gain 20dBu |
| Crosstalk | 100dB ± 5dB @ 0dBu 1Khz |
| Maximum input level | + 18dBu |
| Audio interface standard | 8-way balanced input, phoenix plug |
| Default input and output levels | +0 dBu |
| Microphone preamplifier gain | 0-40 dB analog gain, 12dB digital gain |
| Input impedance | > 5 kΩ balance, > 3 kΩ unbalance |
| Output impedance | 600 Ω |
| Preamplifier equivalent input noise (EIN) | <-125 dBu, 22 Hz-22kHz |
| Output noise floor | -100dBu (A weighted) |
| Phantom power supply | +48 VDC |
| Dimension (WxDxH) | 1U, 480 mm x 250mm x 44mm |
| Power supply | 100 ~ 240VA, 50/60 Hz, 55W |
| Operating temperature range | 0 ~ 40 ° C operating temperature |

# Control software running environment

Operating system requirements: Windows 7 32/64 bit, Windows 10, Windows 11

Network environment: 100M LAN or wireless router

# Control software connect to device

Power on the device, the network cable of the processor and the network cable of the control computer are connected to the same switch.

This digital audio matrix processor uses automatic discovery protocol, and can detect all processors in LAN after opening the software.

When open the software for the first time, the following prompt window will appear. Please check Allow communication on private and public networks, and click to allow access to, otherwise the processor may be found or cannot be connected. The following picture is screenshot  of  Windows7 system.

Windows Security Center

Whether to allow public and private networks to access the application

Windows Firewall has blocked some features of 1608_0808_V1.1.5 on all public and private networks

1608_0808_V1.1.5

Publisher unkown

Path D:\pc_app\1608_0808_v1.15.exe

Allow access to these networks

Public network

Private network

Learn more

Allow

Cancel

# Password Login

Each time the processor is connected, an operating password must be entered in order to operate it, allowing authorised personnel to control the processor's parameters.

Factory default user password: admin

# Login Password Restores Factory Settings

When the login password is lost for some reason, you can use the TCP instruction: PSW_Reset # restore factory password.

Create

Delete

Quit

Connect

Disconnect

Peer IP:192.168.1.128

Peer Port:1698

Local Port:1698

Connected

Socket Status

Date receive window

Date send window

Send Data

Repeat

15:48:23 Send: PSW_Reset#[1]

15:48:23 Receive: PSY:admin

# IP Setting sets the local IP address

When connecting this processor, the IP address of the processor must be in the same network segment as the IP address of the control computer.

For example, as shown in the figure below, the IP address of the processor is 192.168.6.128/255.255.255.0, while that of the controlling computer is 192.168.1.123/255.255.255.0, at this time, the control software can not be connected to the machine, and therefore you need to set the IP address of the processor or the computer's IP address, as shown in the figure below.

Click "Modify Device IP Now", enter a new IP address and click OK. You can modify to the same network segment without restarting. You can also modify the subnet mask if necessary. Here, you can change the device IP to 192.168. 1.128 to log in to the device normally.

# Device List Bar

Control the device list bar on the left side of the software, display and record devices, select login, add offline devices, modify device network parameters, delete devices, disconnect and so on.

11.1. Display device information

In each device, "Device Model", "Device IP Address", "Current Default Name", "Device Error Status Icon" and "Device Online/Offline Icon" are displayed from top to bottom. The "Device Error Status Icon" is not displayed by default, and will be displayed when DSP and other errors occur on the device.

11.2. Connect the login device

Click on the device to be connected online. If the device is connected with the computer network, a password box will pop up. Enter the correct password (the default password is admin) to log in to the device. If the device is disconnected from the computer network, it will prompt the connection to time out.

11.3. Adding device

For some reason, the device is connected to the network normally, but no device is found in the device list. Try adding the device manually. Click the "+" icon in the lower left corner or right-click the mouse in the device list to select Add Device, pop up the above window, and enter the IP address of the device to complete the addition. The IP address of the device is 192.168. 1.128.

11.4. Modify network parameters

You can modify the IP address, subnet mask and gateway of online devices without logging in. Usually, when the computer network and the device network are not in the same network segment when using for the first time, or when prompting the connection timeout when connecting, you can modify the device network parameters here to connect normally.

11.5. Remove devicee

Click the "Remove Device" icon to remove offline devices.

11.6. Device List

Click the "Device List" icon to view the details of the device.

11.7. Disconnect

If you have logged in to a device, double-click the device you are logging in to or click the "Disconnect" icon to log out and disconnect.

# Device Information Status Bar

The status bar at the bottom of the control software displays the current login device information: DSP resource occupancy (real-time usage and peak), serial number and software version. When you click the "System Mute" button, the system will mute. System mute can be operated by "Ctrl + M" shortcut key, and system mute can be controlled by third-party protocol.

The “Lock” icon indicate the PC software to automatically lock down any functions if(device online status),after a certain period of time,no parameters are modified. On locked status, you should click the “Lock” button, input correct password to unlock.

# Input Settings

|  | - Microphone button Press this button when the input is moving coil microphone - Line button Press this button when the input is line level - 48V button Press this button when the microphone is a capacitive microphone and requires 48V phantom power supply - Analog gain When microphone mode is selected, the range is 0 ~ 50dBu, and line mode is 0 ~ 12dBu. Used for amplifying microphone |
| --- | --- |

# Adaptive Feedback Suppressor

This feedback suppressor uses adaptive feedback suppressor, and can improve the sound transmission gain by at least 4 ~ 5dB only by pressing a button.

# AGC Automatic gain

This processor supports independent automatic gain for each channel, and the enable, threshold, target value, slope, startup time and recovery time can be adjusted. Automatic gain parameters can be used to copy and paste channels quickly, which greatly improves the operation efficiency.

# Noise Gate

|  | - Noise gate indicator When the noise gate is opened, the indicator light is off; When the noise gate is closed, the indicator light is on. - Enable button Enable or close the noise gate - Threshold Determines how much of the input signal opens after the noise gate,i.e. after what input signal the sound signal can pass through. Range-120 ~ 0dB. - Startup time How long does the noise gate open after the input signal exceeds the threshold, that is, the time required for the noise gate to open from closing to opening. - Recovery time How long does it take for the noise gate to close after the input signal is less than the threshold, that is, the time required from opening to closing. |
| --- | --- |

# Input Filter

This processor supports independent filters for each channel, with 8-band PEQ per channel. The input filters include various professional audio filtering modules: high-pass, low-pass, parametric equalization, low-frame equalization and high-frame equalization.

Filter parameters can be copied and pasted quickly, which greatly improves the operation efficiency.

# Input Limiter

This processor supports an independent input limiter for each channel, and the activation, threshold, slope, startup time and recovery time can be adjusted. The limiter parameter can be copied and pasted quickly, which greatly improves the operation efficiency.

# Pre-Mix Volume

Before Matrix mixing, it can monitor the post-fader level of each channel, control the volume, and group, reverse and mute the input audio signals.

The volume faders of this page can be controlled through a third-party protocol.

# Matrix Mixer

Matrix mixing supports any cross mixing, and multi-channel selection can be carried out by holding down the left mouse button. Click the edit button in the upper left corner to set the mixing volume. Click on the input channel name to mix a single input channel to all output channels. Click on the output channel name to mix all input channels to the output channel.

# Auto Mix

Automatic mixing algorithm is usually used in multi-microphone conference system, which can effectively avoid howling when multiple microphones are turned on at the same time.

Instructions for using automatic mixing

1. Have two separate auto-mix groups.

2. Select AM 1 or AM2 button to assign the channel to Auto Mix Group 1 or 2.

3. Use a fader to modify the priority of the volume assigned to the channel.

Usually, which channel's sound is important, its weight is distributed more. Typically, the default value is 0.

4. Click the AM X Enable button to enable or disable Auto Mix Group X.

5. The upper level indicates the attenuation amount, and the lower level indicates the attenuated level.

# Signal Generator

The signal generator supports pink noise, sine wave and white noise, which can be distributed to any output.

# Output Volume

After Matrix Mix, it can monitor the fader level of each channel, control the volume, and group, invert and mute the input audio signals.

The volume faders of this page can be controlled through a third-party protocol.

# Output Filter

This processor supports independent filters for each channel, with 8-band PEQ per channel. The output filter includes various professional audio filtering modules: high-pass, low-pass, PEQ, low-frame equalization and high-frame equalization.

Filter parameters can be copied and pasted quickly, which greatly improves the operation efficiency.

# Output Limiter

The processor supports an independent output limiter for each channel, and the activation, threshold, slope, startup time and recovery time can be adjusted. The limiter parameters can be copied and pasted quickly, which greatly improves the operation efficiency.

# Output Delay

Each output channel has an independent delay, and the maximum delay time can be set to 300 milliseconds.

# Presets

This processor can store 32 presets, and each mode can be named by itself.

Presets can be called through control software or through open instructions to third-party devices.

Default parameters support data import and export.

# Setup settings

28.1. Device Information

■ Device model

■ ARM Version: ARM firmware version number.

■ DSP version: DSP firmware version number.

■ Serial number: the factory serial number of the device or the batch code being produced.

■ Running time: The running time after the device is powered on, and it is timed from 0 again after the power is cut off.

■ USB status: unplugged in / plugged in.

28.2. Device Status

The indicator light is gray and unknown; The indicator light is green and normal; The indicator light is red and flashing, which is abnormal.

28.3. Device Setup

■ Device ID

■ Device location: Mark the device location.

■ Device Description: Describes device information.

■ IP Address, Subnet Mask, Gateway: Set network parameters.

■ Noise gate spit code: whether to open the noise gate central control code.

■ RS232 baud rate: modify RS232 baud rate.

■ RS485 baud rate: this parameter has been solidified and cannot be modified.

■ Fan test: Control the fan switch.

28.4. General Settings

■ Login password: Set the password of PC software or third-party central control connection login device

■ Login password: Set the PC software to automatically lock down any functions if, after a certain period of time,no parameters are modified.

28.5. Gain Limit

Set the allowable gain setting range of each channel.

# Record/Play

- USB playback supports mp3, wav, ape, aac and flac file formats; The recording file format is wav.

- USB storage device only supports FAT32 format. Playback files are fixed stored in the audio folder of the root directory, and recording files are fixed stored in the record folder of the root directory.

- Play start, pause, stop, previous song, next song, play mode.

- Recording starts/stops.

# Label

On the tab page, you can manage the tags of input and output channels, and the tags support Chinese input. After the label input is completed, when the mouse rests on the corresponding volume putter or channel button, the content of the channel label will be displayed in suspension.

# Meter level

On the Level page, monitor the levels before and after all input and output channel faders in real time.

# Device List

Displays each device IP address, device model, ARM version, DSP version, device location, device description, serial number, network connection status (offline/online unlogged/online logged/logged), run time, and device ID.

# Command Assistant

Used to generate Hex format commands.

# Communication protocol / third party Communication protocol-ASCII character format

| Communication protocol type: TCP sever port number: 2698 (no login required) RS232 default baud rate: 115200 |  |  |  | When instructions are sent from third-party devices, it is recommended that each instruction be delayed by 50ms. |  |
| --- | --- | --- | --- | --- | --- |
| Serial No. | Function | TCP communication protocol | RS-232 protocol | Feedback code | Remarks |
| 1 | Modify device location | Set_Loc [QC_Test_1] # | No | Location: QC_Test_1 | The characters in brackets are ASCII characters, which must be free of commas and other special characters, limited to 16 characters, and LCD screens with more than 16 characters cannot be displayed; |
| 2 | Modify device description | Set_Des [QC_Test_2] # | No | Description: QC_Test_2 |  |
| 3 | Recall the preset | LOADP X# | LOADP X# | Preset: 1 | LOADP 1 # Description: Loads and activates a default. Variable X represents the preset number, and variable range 1 ~ 32 channel, 1 represents the default preset for startup |
| Serial No. | Function | TCP communication protocol | RS-232 protocol | Feedback code | Remarks |
| 4 | Read the current preset | ReadP # | ReadP # | Preset: 0 Preset_Name: Boot Preset |  |
| 5 | Invalid system mute control | SYM 0# | SYM 0 # | SYM 0 # | Parameter 0 indicates that system mute is invalid. Parameter 1 indicates that system mute is valid and all outputs are muted |
|  | System mute control is effective | SYM 1# | SYM 1 # | SYM 1 # |  |
| 6 | Read MatrixMix volume before mixing | ReadL1 X# | ReadL1 X# | PreLevel X: 0dB | Variable X is audio channel, variable range 1~ 16 channel. Y is volume dB value, variable range: +12 ~-71 single step increment (decrement) is 1dB |
| 7 | Volume increment before mixing | L1_add X # | L1_add X # | PreLevel X:-20dB |  |
| 8 | Volume decrease before mixing | L1_sub X # | L1_sub X # | PreLevel X:-20dB |  |
| 9 | Volume increment after mixing | L2_add X # | L2_add X # | PostLevel X: 0dB |  |
| 10 | The volume decreases after mixing | L2_sub X # | L2_sub X # | PostLevel X: 0dB |  |
| 11 | Set the volume before MatrixMix mixing | SetL1 X:Y# SetL1 1:-20# | SetL1 X:Y# | PreLevel X:-20dB |  |
| 12 | Read the volume after MatrixMix mixing | ReadL2 X# ReadL2 16# | ReadL2 X# | PostLevel X: 0dB |  |
| 13 | Set the volume of MatrixMix after mixing | SetL2 X: Y# SetL2 1:-20# | SetL2 X:Y# | PostLevel X:-20dB |  |
| Serial No. | Function | TCP communication protocol | RS-232 protocol | Feedback code | Remarks |
| 14 | Read noise gate status output | Read_Gate # | Read_Gate # | 0100000000000000 | 0 ~ 1: noise gate state, 0 denotes that the noise gate is not opened, and 1 denotes that the noise gate is open. For example, 010000000000000000 marks the channel 2 Noise gate is open |
| 15 | Password reset | PSW_Reset # |  | PSW: admin | The password after reset is admin |
| 16 | RS-232 Reset IP Address |  | IP_Reset # | IP_Reset OK | IP Address: 192.168. 1.128 Subnet Mask: 255.255. 255.0 Gateway: 192.168. 1.1 |
| 17 | Channel mute before mixing | L1_Mute X # | L1_Mute X# | L1_Mute X | The variable X is audio channel, and the variable range is 1 ~ 16 channel |
|  | Channel de-mute before mixing | L1_UnMute X# | L1_UnMute X# | L1_UnMute X |  |
| 18 | Channel mute after mixing | L2_Mute X # | L2_Mute X# | L2_Mute X |  |
|  | After mixing, the channel is unmuted | L2_UnMute X# | L2_UnMute X# | L2_UnMute X |  |
| 19 | Channel mute state before reading mix | ReadL1 Mute # | ReadL1 Mute # | L1Mute: 00001111000000000 L1Mute: 000000000000000000 | Parameter 0 indicates that channel mute is invalid. Parameter 1 indicates that channel mute is valid and sound output is prohibited |
| 20 | Channel mute state after reading mix | ReadL2 Mute # | ReadL2 Mute # | L2Mute: 00001111000000000 L2Mute: 000000000000000000 |  |
| Serial No. | Function | TCP communication protocol | RS-232 protocol | Feedback code | Remarks |

# Third party communication protocol: TCP HEX format

## 35.1 Data transmission format

Read data

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) |  | Data (n byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Low | High |  |  |
| 0x7D82 |  |  |  |  | 0x00 |  |  |  | 0x827D |

Write data

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) |  | Data (n byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Low | High |  |  |
| 0x7D82 |  |  |  |  | 0x01 |  |  |  | 0x827D |

## 35.2 Device Return Data Format

Data normal response back code

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (n byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  |  |  |  | 0x80 |  |  | 0x827D |

Instruction parameter error code return

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  |  |  |  | 0x81 | 0x0000 | 0x827D |

The instruction does not recognize the return code

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  |  |  |  | 0x82 | 0x0000 | 0x827D |

## 35.3 Examples

| Serial No. | Function | TCP communication protocol | Remarks |
| --- | --- | --- | --- |
| 1 | Login | 82 7D 01 00 02 01 01 01 05 00 61 64 6D 69 6E 7D 82 | Default password: admin |
| 2 | Channel mute before mixing | 82 7D 01 00 03 07 01 01 02 00 00 01 7D 82 82 7D 01 00 03 07 01 01 02 00 00 00 7D 82 | Input 1 Mute Input 1 Unmute |
| 3 | Channel gain before mixing | 82 7D 01 00 03 07 02 01 05 00 00 00 40 C0 7D 82 | Channel 1 gain is -3. 0dB |
| 4 | Channel gain accumulation/subtraction before mixing | 82 7D 01 00 03 07 05 01 05 00 00 00 80 3F 7D 82 82 7D 01 00 03 07 05 01 05 00 00 00 80 BF 7D 82 | Input 1 Gain +1 Input 1 Gain-1 |
| 5 | Channel mute after mixing | 82 7D 01 00 03 0B 02 01 02 00 00 01 7D 82 82 7D 01 00 03 0B 02 01 02 00 00 00 7D 82 | Output 1 Mute Output 1 Unmute |
| 6 | Channel gain after mixing | 82 7D 01 00 03 0B 03 01 05 00 00 00 40 40 7D 82 | Output 1 gain 3.0 dB |
| 7 | Channel gain accumulation/subtraction after mixing | 82 7D 01 00 03 0B 06 01 05 00 00 00 80 3F 7D 82 82 7D 01 00 03 0B 06 01 05 00 00 00 80 BF 7D 82 | Output 1 Gain +1 Output 1 Gain-1 |
| 8 | Load preset | 82 7D 01 00 03 20 03 01 01 00 04 7D 82 | Load preset 5 |
| 9 | Pre-mixing channel level meter | 82 7D 01 00 03 01 01 00 00 00 7D 82 | Level after reading input channel fader Feedback code: 82 7D 01 00 03 01 01 80 10 00 (16 bytes of input 1-16 level values) 7D 82 |
| 10 | After mixing channel level meter | 82 7D 01 00 03 0B 01 00 00 00 7D 82 | Level after reading output channel fader Feedback code: 82 7D 01 00 03 0B 01 80 10 00 (16 bytes out of 1-16 level values) 7D 82 |

Please refer to the instruction assistant for more information.

# Mixer Matrix TCP/IP, RS-232 Control Instruction

This protocol supports TCP and RS232 control. The TCP port number is 1698 or 1699 or 2698, where the use of port 2698 requires login first, and port 1698 or 1699 does not require login. The default baud rate of RS232 is 115200.

Default password TCP login password: admin, all instructions need to log in before it can be controlled.

The default encoding is 1 (that is, 0x0001).

### 36.1 Control Mixer State (Function ID: 01H [R/W])

Read data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (1byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x01 | 0x00 | 0x0001 | Output channel | 0x827D |

Write data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (n byte) |  |  |  |  | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x01 | 0x01 |  | Output Channel | Input Channel | … | Input Channel | Status | 0x827D |
| Data in the first byte for the output channel, the second to n-1 bytes for the input channel, the last byte for the status byte, 0x00 denotes the cancellation of the mix, 0x01 denotes the mix. For example: Mix Input 1, 2, 3 channels to Output 1, the data sent is: 0x82 0x7D 0x01 0x00 0x03 0x08 0x01 0x01 0x01 0x05 0x00 0x00 0x00 0x01 0x02 0x01 0x7D 0x82 |  |  |  |  |  |  |  |  |  |  |  |  |

Device return data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (17 byte) |  |  |  | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x01 | 0x80 | 0x0011 | Output Channel | Enter 1 Channel state | … | Enter 16 Channel state | 0x827D |

### 36.2 Control Mixer Value (Function ID: 02H [R/W])

Read data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (1byte) | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x02 | 0x00 | 0x0001 | Output channel | 0x827D |

Write data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (n byte) |  |  |  |  | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x02 | 0x01 |  | Output Channel | Input Channel | … | Input Channel | Mix value (1byte) | 0x827D |
| Note: The first byte in the data is the output channel, the second to n-1 byte is the input channel, and the last byte is the mixing value, ranging from-70 to 12dB, and-70 means mute. For example, if the mix value of input 1, 2, 3 channels to output 1 is set to-10dB, the transmit data is: 0x82 0x7D 0x01 0x00 0x03 0x08 0x02 0x01 0x05 0x00 0x00 0x00 0x01 0x02 0xF6 0x7D 0x82 |  |  |  |  |  |  |  |  |  |  |  |  |

Device return data format

| Frame header (2byte) | Code (2byte) | Type (1byte) | Subclass (1byte) | Function ID (1byte) | Command (1byte) | Data length (2byte) | Data (17 byte) |  |  |  | Terminator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x7D82 |  | 0x03 | 0x08 | 0x01 | 0x80 | 0x0011 | Output Channel | Enter 1 Mix value | … | Enter 16 Mix value | 0x827D |

# Noise gate spit code and camera linkage

In the device settings page, there is a noise gate spit code control, which defaults to No status.

When switching to Yes state, Gate will automatically send out a series of codes to realize camera linkage triggering.

Realization mode of camera linkage

Mode 1:

This method is suitable for central control brands with inflexible programming,

Noise Gate in microphone mode, all 16 channel thresholds are adjusted to -15dBu (adjusted according to the situation on site). Find the following buttons in the software interface: Settings/ Device Information/Noise Gate Active Code Spitting/NO.

The active code spitting function of microphone noise gate is closed by default, which needs to be set to YSE and take effect immediately. After restarting, it is in YES state.

For example:

The second microphone is turned on, and the processor sends out the code AA 02 01 0D; (AA: prefix; 02: first microphone; 01 microphone on; 0D: end code)

The second microphone is turned off, and the processor sends out the code AA 02 00 0D; (AA: prefix; 02: first microphone; 00 microphone off; 0D: end code)

When the first and third microphones are turned on, and then the sixth microphone is turned on, the code sending sequence is as follows:

When the first microphone is turned on, the processor issues the code AA 01 01 0D

When the second microphone is turned on, the processor issues the code AA 02 01 0D

..

When the sixth microphone is turned on, the processor issues the code AA 06 01 0D

..

When several microphones are turned on at the same time, central control programming is needed to judge which microphone is continuously speaking (using the closing instruction);

Because the processor can't judge whether the microphone is talking normally or excited by teacups and coughing sounds.

Mode 2:

This method is suitable for AMX and Crestron brands.

The central control host needs to circulate 50ms to actively send Read_Gate #, and the audio processor will return "10100000000001".

Noise Gate in microphone mode, threshold adjusted to -15dBu (adjusted according to situation).

The central control host needs to send codes to the processor before knowing the status of the microphone.

For example, the read microphone status code is "10100000000001", which means that three microphones are opened (i.e., the first, third and sixteenth). If the camera is automatically tracked, the read three microphones are all open, which needs the central control host to identify, and the central control needs to judge which microphone is continuously speaking

Because the processor can't judge whether the microphone is talking normally or excited by teacups and coughing sounds.

# System Upgrade System Upgrade

This processor can upgrade firmware through Ethernet. The operation steps are as follows.

Click "System Upgrade" in the menu bar, and a dialog box will pop up. Select the firmware with img suffix, click OK, and the upgrade progress bar will appear. After the upgrade is completed, the device will restart automatically.

# System Reset

After connecting to the login processor, you can clear all parameters by clicking "System Reset" in the menu bar.

# Common Troubleshooting

## 40.1 The control software could not find the processor

The functions of the control software are blocked by Windows Firewall, or some functions are disabled by antivirus software on your computer, which causes it to not work properly.

Therefore, run the exe file of the processor with system administrator privileges.

If the antivirus software directly deleted or isolated part of the software files, please release the isolated files.

## 40.2 The control software found the device but could not establish a connection

The control software works in a dual network environment, such as a notebook computer connected to a wired network and a wireless router. This may occur at this time. Disabling another network will solve this problem.

## 40.3 PC Control Software Failed to Modify IP Address

Because this audio processor uses UDP protocol for IP address setting, if you use a wireless router to connect to the processor, the wireless router will block the UDP protocol of the audio processor, resulting in no modification. You need to use a notebook computer to establish a direct connection with the audio processor for modification.

## 40.4 Third Party TCP Instructions Unable to Control

A. Once the third-party TCP instruction control is set, the IP address of the host cannot be changed arbitrarily, otherwise the control is invalid.

B, please check whether the control port is correct;

C, please check whether the control code is accurate;

D. If the above problems cannot be solved, please use the general TCP instruction debugging tool for testing. For specific operation methods, please refer to "Video Guide for Communication Protocol Testing"